Name                 : Nikhitha V

Reg No              :192512490

Course name     : Advanced Wireless Commincation

Course code      : ECA5610

MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular Interference

Aim:

# To compare a simplified free-space path-loss estimate across representative sub-1 GHz, mid-band and mmWave frequencies, and examine how distance and frequency reuse affect received signal and a simple interference measure

# Objectives:

1.  Calculate or simulate path loss at representative sub-1 GHz, mid-band and mmWave frequencies.

2.  Compare estimated received signal strength as frequency and distance change.

3.  Examine how cell separation or frequency reuse changes a simplified interference ratio.

4.  Relate the results to coverage and spectrum-use considerations across mobile generations, while stating model limitations.

# Requirements:

  - Computer with MATLAB

# Program :

clear; clc; close all;

% Representative bands and link-budget assumptions

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

txPowerdBm = 30;

gtDbi = 0; grDbi = 0;

distanceKm = logspace(log10(0.1),log10(2),200);

distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz,distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName,freqGHz(:),lossAtPoint(:),rxAtPoint(:), ...

'VariableNames',{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'});

disp(linkTable);

% Path loss and received power over distance

figure; tiledlayout(1,2);

nexttile; hold on; grid on;

for k=1:numel(freqMHz)

plot(distanceKm,fspl(freqMHz(k),distanceKm),'LineWidth',1.5);

end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Path loss (dB)');

legend(bandName,'Location','northwest'); title('Free-Space Path Loss');

nexttile; hold on; grid on;

for k=1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k),distanceKm);

plot(distanceKm,rx,'LineWidth',1.5);

end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Estimated received power (dBm)');

legend(bandName,'Location','southwest'); title('Illustrative Received Power');

% Simple first-tier co-channel reuse estimate

cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6;

sirDb = 10*log10(sirLinear);

reuseTable = table(reuseN(:),reuseDistance(:),sirDb(:), ...

'VariableNames',{'ReuseClusterN','ApproxInterfererDistance_km','ApproxSIR_dB'});

disp(reuseTable);

figure; plot(reuseN,sirDb,'o-','LineWidth',1.5); grid on;

xlabel('Reuse cluster size N'); ylabel('Approximate SIR (dB)');

title('Simplified Co-Channel SIR versus Reuse Cluster Size');

fprintf('At %.2f km, estimated received powers are:\n',distanceAtPointKm);

disp(table(bandName,rxAtPoint(:),'VariableNames',{'Band','Rx_dBm'}));

# Output:

+

# Conclusion

Summarize the frequency and distance trends from the simulation and the effect of reuse separation on the illustrative SIR. Identify the free-space assumption, fixed transmit power and idealized six-interferer geometry as limitations. Note that practical coverage depends on antenna characteristics, environment, bandwidth, receiver sensitivity, interference, regulatory constraints and the chosen propagation model.